02 — MEMORY ARCHITECTURE: BRAM vs DDR

1. NGUYÊN TẮC

BRAM = hot, bounded, deterministic working state.

DDR = large, cold/warm, long-term evidence/episode/skill/checkpoint.

Không giữ raw history lớn trong BRAM.

2. TÀI NGUYÊN THAM CHIẾU ARTY A7-100T

XC7A100T: 63,400 LUT, 126,800 FF, 240 DSP48E1, 135 BRAM36 ≈ 4,860 Kbit ≈ 607.5 KiB. Board có DDR3 256 MiB-class. Đây là ceiling vật lý; full integration phải chừa margin cho MIG, routing, FIFO, reset/CDC.

3. HOT STATE NÊN Ở BRAM / DISTRIBUTED RAM

- current RSR / active goal / legal mask

- Q* weight bank A/B

- SPEAR weights + small uncertainty/counter stats

- active candidate set

- 16–32 step trajectory

- pending reward identity

- current skill descriptor

- ASTRA proof scratch / Top-K hot cache

- 32–64 recent failure prototypes

- compactor staging

- UART/FIFO buffers

- small GEMINI token/symbol buffer

Q*/SPEAR matrices rất nhỏ; thường chỉ cần distributed RAM hoặc 1–2 BRAM tổng.

4. LONG-TERM NÊN Ở DDR

- ASTRA evidence directory/postings/facts/provenance

- knowledge packs

- full skill library

- episodic journal

- raw failure journal

- compacted failure prototypes

- procedural memory

- lexicon/dictionaries lớn

- policy checkpoints A/B

- experiment snapshots

- cold proof/candidate data

5. BRAM ENGINEERING BUDGET CANDIDATE

ASTRA proof/retrieval hot cache: 18–28 BRAM36

Working memory/RSR/candidate: 4–8

Trajectory/pending/return: 2–4

Skill hot cache: 2–4

Failure hot cache/compactor: 2–4

GEMINI token buffers: 4–8

UART/protocol/FIFO: 2–4

MIG user-side buffering: 8–16

Q*/SPEAR weights/stats: 0–2

Preferred spare/routing relief: >=20

Không coi bảng này là utilization claim. Chỉ OOC/full synth mới chốt.

6. DDR MAP V2 COMPACT CANDIDATE — KHÔNG PHẢI LỆNH MIGRATE V1

0x0000_0000–0x003F_FFFF   4 MiB   system manifests + dual checkpoint headers

0x0040_0000–0x00FF_FFFF  12 MiB   lexicon/codebook/skill metadata

0x0100_0000–0x04FF_FFFF  64 MiB   packed evidence directory + postings

0x0500_0000–0x08FF_FFFF  64 MiB   facts + provenance + semantic packs

0x0900_0000–0x0BFF_FFFF  48 MiB   episodic + raw failure journal

0x0C00_0000–0x0CFF_FFFF  16 MiB   compacted skill/failure/procedural memory

0x0D00_0000–0x0DFF_FFFF  16 MiB   policy checkpoints + experiment scratch

0x0E00_0000–0x0FFF_FFFF  32 MiB   spare / legacy compatibility / growth

Total = 256 MiB.

7. VÌ SAO V2 NÊN EVALUATE PACKED/PAGED POSTINGS

V1 fixed-slot posting reservation cho deterministic addressing nhưng tốn address space. V2 nên benchmark packed/paged postings với invariant:

TOTAL_EVIDENCE tăng nhưng DDR bytes/query, candidate count và proof work vẫn bounded.

Không đổi V1 address ABI trước migration gate riêng.

8. CHECKPOINT ATOMICITY

Dùng slot A/B:

write inactive slot → CRC/SHA → verify → atomically flip active generation pointer.

Không overwrite active checkpoint in place.

Checkpoint tối thiểu:

- Q*/SPEAR weights

- policy versions

- skill-table version

- failure prototype table

- compaction counters

- allocator state

- capability manifest version

- corpus epoch

9. SHARED ARITHMETIC

Q* eval, SPEAR rank, n-step return và weight update không cần chạy đồng thời. Time-multiplex shared sequential MAC để tránh DSP/LUT explosion.

PHASE_Q_EVAL → PHASE_SPEAR → PHASE_EXEC → PHASE_RETURN → PHASE_UPDATE.